COMPUTER NETWORKS LAB

WEEK 1

NAME: VISHWA MEHUL MEHTA

SRN: PES2UG20CS389

SECTION: F

DATE: 21/01/2022

Study and understand the basic networking tools - Wireshark, Tcpdump, Ping, Traceroute and Netcat.

Task 1: Linux Interface Configuration (ifconfig / IP command)

Step 1: To display status of all active network interfaces.

ifconfig (or) ip addr show

Analyze and fill the following table:

ip address table:

| Interface name | IP address (IPv4 / IPv6) | MAC address |  |
| --- | --- | --- | --- |
| lo | 127.0.0.1 | 00:00:00:00:00:00 | Loop back device |
| enp0s3 | 10.0.2.15 | 08:00:27:4f:0b:16 | Ethernet |
| WLAN | - | - | - |

Step 2: To assign an IP address to an interface, use the following command. sudo ifconfig interface_name 10.0.your_section.your_sno netmask 255.255.255.0 (or) sudo ip addr add 10.0.your_section.your_sno /24 dev interface_name

Step 3: To activate / deactivate a network interface, type.

sudo ifconfig interface_name down

sudo ifconfig interface_name up

Step 4: To show the current neighbor table in kernel, type

ip neigh

Task 2: Ping PDU (Packet Data Units or Packets) Capture

Step 1: Assign an IP address to the system (Host).

Note: IP address of your system should be 10.0.your_section.your_sno.

Step 2: Launch Wireshark and select ‘any’ interface

Step 3: In terminal, type ping 10.0.your_section.your_sno

Observations to be made

Step 4: Analyze the following in Terminal

• TTL : 64

• Protocol used by ping : ICMP (Internet Control Message Protocol)

• Time : 18418ms

Step 5: Analyze the following in Wireshark

On Packet List Pane, select the first echo packet on the list. On Packet Details Pane, click on  each of the four “+” to expand the information. Analyze the frames with the first echo request  and echo reply and complete the table below.

First echo request:

First echo reply:

| Details | First Echo Request | First Echo Reply |
| --- | --- | --- |
| Frame Number | 1 | 2 |
| Source IP address | 10.0.6.56 | 10.0.6.56 |
| Destination IP address | 10.0.6.56 | 10.0.6.56 |
| ICMP Type Value | 8 | 0 |
| ICMP Code Value | 0 | 0 |
| Source Ethernet Address | 00:00:00:00:00:00 | 00:00:00:00:00:00 |
| Destination Ethernet Address | 00:00:00:00:00:00 | 00:00:00:00:00:00 |
| Internet Protocol Version | 4 | 4 |
| Time To Live (TTL) Value | 64 | 64 |

Task 3: HTTP PDU Capture

Using Wireshark’s Filter feature

Step 1: Launch Wireshark and select ‘any’ interface. On the Filter toolbar, type-in ‘http’ and  press enter

Step 2: Open Firefox browser, and browse www.flipkart.com

Observations to be made

Step 3: Analyze the first (interaction of host to the web server) and second frame (response  of server to the client). By analyzing the filtered frames, complete the table below:

HTTP First Request:

HTTP First Response:

| Details | First Echo Request | First Echo Reply |
| --- | --- | --- |
| Frame Number | 35 | 38 |
| Source Port | 38282 | 80 |
| Destination Port | 80 | 38282 |
| Source IP address | 10.0.6.56 | 34.107.221.82 |
| Destination IP address | 34.107.221.82 | 10.0.6.56 |
| Source Ethernet Address | 08:00:27:4f:0b:16 | 52:54:00:12:35:02 |
| Destination Ethernet Address | 52:54:00:12:35:02 | 08:00:27:4f:0b:16 |

Step 4: Analyze the HTTP request and response and complete the table below.

| HTTP Request |  | HTTP Response |  |
| --- | --- | --- | --- |
| Get | GET /Success.txt?ipv4 HTTP/1.1 | Server | nginx\r\n |
| Host | detectportal.firefox\r\n | Content-Type | text/plain\r\n |
| User-Agent | Mozilla/5.0 (X11; Ubuntu; Linux x86_64; rv:96.0) Gecko/20100101 Firefox/96.0\r\n | Date | Thu, 20 Jan 2022 16:25:30 GMT\r\n |
| Accept-Language | en-US,en;q=0.5\r\n | Location | - |
| Accept-Encoding | gzip, deflate\r\n | Content-Length | 8\r\n |
| Connection | keep-alive\r\n | Connection | close\r\n |

Using Wireshark’s Follow TCP Stream

Step 1: Make sure the filter is blank. Right-click any packet inside the Packet List Pane, then  select ‘Follow TCP Stream’. For demo purpose, a packet containing the HTTP GET request  “GET / HTTP / 1.1” can be selected.

Step 2: Upon following a TCP stream, screenshot the whole window.

Task 4: Capturing packets with tcpdump

Step 1: Use the command tcpdump -D to see which interfaces are available for capture. sudo tcpdump -D

Step 2: Capture all packets in any interface by running this command:

sudo tcpdump -i any

Note: Perform some pinging operation while giving above command. Also type  www.google.com in browser.

Observation

Step 3: Understand the output format.

Step 4: To filter packets based on protocol, specifying the protocol in the command line. For  example, capture ICMP packets only by using this command:

sudo tcpdump -i any -c5 icmp

Step 5: Check the packet content. For example, inspect the HTTP content of a web request  like this: sudo tcpdump -i any -c10 -nn -A port 80

Step 6: To save packets to a file instead of displaying them on screen, use the option -w: sudo tcpdump -i any -c10 -nn -w webserver.pcap port 80

Task 5: Perform Traceroute checks

Step 1: Run the traceroute using the following command.

sudo traceroute www.google.com

Step 2: Analyze destination address of google.com and no. of hops

Destination: 142.250.196.36    No. of hops: 30

Step 3: To speed up the process, you can disable the mapping of IP addresses with hostnames  by using the -n option

sudo traceroute -n www.google.com

Step 4: The -I option is necessary so that the traceroute uses ICMP.

sudo traceroute -I www.google.com

Step 5: By default, traceroute uses icmp (ping) packets. If you’d rather test a TCP connection  to gather data more relevant to web server, you can use the -T flag.

sudo traceroute -T www.google.com

Task 6: Explore an entire network for information (Nmap)

Step 1: You can scan a host using its host name or IP address, for instance. nmap www.pes.edu

Step 2: Alternatively, use an IP address to scan.

nmap 163.53.78.128

Step 3: Scan multiple IP address or subnet (IPv4)

nmap 192.168.1.1 192.168.1.2 192.168.1.3

TASK 7 A): NETCAT AS CHAT TOOL
a) Intra system communication (Using 2 terminals in the same system)
Step 1: Open a terminal (Ctrl+Alt+T). This will act as a Server.
Step 2: Type nc -l any_portnum (For eg., nc -l 1234)
Note: It will goto listening mode
Step 3: Open another terminal and this will act as a client.
Step 4: Type nc <your-system-ip-address> portnum
Note: portnum should be common in both the terminals (for eg., nc 10.0.2.8 1234)
Step 5: Type anything in client will appear in server

Questions on above observations:

1) Is your browser running HTTP version 1.0 or 1.1? What version of HTTP is the  server?

HTTP 1.1 for browser and server.

2) When was the HTML file that you are retrieving last modified at the server?

Thu, 20 Jan 2022 16:25:30 GMT\r\n

3) How to tell ping to exit after a specified number of ECHO_REQUEST packets?

ping ip –v no_of_pings

4) How will you identify remote host apps and OS?

nmap –O –v ip_addr